GHANA ROBOTICS COMPETITION 2026

STARS LEAGUE

GAME MANUAL

The Green Recovery Mission

Operation Green Recovery

10 Missions • 4-Quadrant Field • 2v2 Alliance Play

2 Minutes Autonomous + 1 Minute Manual

Theme: AI & Robotics for Sustainable Africa

Field: 96 × 96 inches (2438 × 2438mm)

Version 3.0 — April 2026

Prepared by Firefly IO — grc@fireflyio.com

# 1. Competition Overview

## 1.1 Match Format

| Parameter | Value |
| --- | --- |
| Match Duration | 3 minutes total |
| Autonomous Period | 2 minutes (robots in own quadrant) |
| Manual Control Period | 1 minute (full field opens) |
| Endgame | Last 30 seconds of manual |
| Alliance Format | 2v2 (Red vs Blue) |
| Robots per Alliance | 2 (each in own quadrant) |
| Total Missions | 10 (4 per robot + 2 shared center) |
| Field Size | 96 × 96 inches (2438 × 2438mm) |
| Robot Platform | XploreBot |
| Robot Start Size | 30 × 30 × 30 cm max |

## 1.2 The Quadrant System

The field is divided into 4 quadrants. Each robot gets its own quadrant with its own Home Base and missions. During the 2-minute autonomous period, all 4 robots work independently in their own quadrants — no collision risk. During the 1-minute manual period, the field opens, and alliance partners can cross quadrants and access the shared center.

| Quadrant | Assignment |
| --- | --- |
| Top-Left | Red Robot 1 — Mine Safety + Soil + Wildlife + Accelerator |
| Top-Right | Blue Robot 1 — Mine Safety + Soil + Wildlife + Accelerator |
| Bottom-Left | Red Robot 2 — Reforestation + Weather + Seeds + Galamsey |
| Bottom-Right | Blue Robot 2 — Reforestation + Weather + Seeds + Galamsey |
| Center | Shared — AI Command Center + Water Purification (manual only) |

Alliance partners are on the same side (adjacent quadrants). Scoring: R1 + R2 + center = Red Alliance total.

## 1.3 Track System

The field uses a hybrid track layout: main trunk lines with numbered branches and cross-links. Primary routes are solid black lines (20mm wide). Cross-links are dashed lines for alternate return paths. Primary junctions are filled green circles. Cross-link junctions are hollow green circles.

## 1.4 Challenge Narrative

Illegal mining has scarred Ghana’s landscape. Open pits endanger communities, soil is contaminated, wildlife is displaced, forests are destroyed, weather systems are down, and seeds of native plants are at risk. An AI-powered command center must be activated to coordinate the recovery. Two robots per alliance tackle different aspects of the crisis independently during autonomous, then come together during manual to complete the command center and purify contaminated water.

# 2. Robot 1 Missions — Mine & Wildlife Recovery

Robot 1 operates in the top quadrant. These missions focus on mine safety, soil testing, wildlife rescue, and activating the alliance’s Resilience Accelerator.

## M1. Mine Pit Safety

Illegal mining has left dangerous open pits. Robots pick up color-coded safety containers from a track line and deliver each to its matching colored pit.

How It Works

Four safety containers (color-coded) are placed along a horizontal track line in the quadrant. Four matching mine pits are located at the top edge. The robot navigates to a container, picks it up, then follows the track to the matching pit. Placing a container in the wrong pit incurs a penalty.

Scoring

| Action | Points |
| --- | --- |
| Correct container in matching pit (Autonomous) | 40 |
| Correct container in matching pit (Manual) | 10 |
| Wrong color container in pit | -15 penalty |
| All 4 pits correctly secured | +30 completion bonus |

Field Elements

| Element | Specification |
| --- | --- |
| Mine Pits | 4 recessed zones at top edge of quadrant (color-coded) |
| Safety Containers | 4 cubes (50mm), spaced along horizontal track line |

## M2. Soil Testing Station

Before land can be replanted, soil contamination must be assessed. The robot pushes a lever on the soil testing station to flip it and reveal the soil sample result.

Scoring

| Action | Points |
| --- | --- |
| Lever fully flipped / latched (Autonomous) | 30 |
| Lever fully flipped / latched (Manual) | 10 |

Field Elements

| Element | Specification |
| --- | --- |
| Soil Testing Station | Hinged lever mechanism on track line, side edge of quadrant |

• No-equipment constraint: robot cannot be touching the station at match end to score.

## M3. Wildlife Rescue

Environmental damage has displaced wildlife into a danger zone. The robot picks up an animal figure and delivers it safely back to the Home Base (which serves as the wildlife sanctuary).

Scoring

| Action | Points |
| --- | --- |
| Animal delivered to Home Base (Autonomous) | 35 |
| Animal delivered to Home Base (Manual) | 10 |

Field Elements

| Element | Specification |
| --- | --- |
| Animal Figure | 1 per quadrant, in marked Danger Zone on track line |
| Sanctuary | Home Base — animal must be fully inside Home to score |

## M4. Resilience Accelerator

Each alliance has its own Resilience Accelerator button in their Robot 1 quadrant. When activated, it doubles the alliance’s AI Command Center scoring for the remainder of the match.

Scoring

| Action | Points |
| --- | --- |
| Accelerator activated | 0 (enables 2× M9 Command Center scoring) |

Rules

• Each alliance has its own Accelerator — not shared between alliances

• Activation window: between 1:45 and 1:40 of the Autonomous  period

• Robot must press and hold for approximately 2 seconds

• Once activated, doubles all AI Command Center scoring for that alliance

# 3. Robot 2 Missions — Land & Climate Recovery

Robot 2 operates in the bottom quadrant. These missions focus on reforestation, weather monitoring, seed preservation, and clearing illegal mining equipment.

## M5. Reforestation

Degraded mining land must be replanted. The robot collects tree seedlings from the Tree Nursery and plants them in 3 separate planting depots.

Scoring

| Action | Points |
| --- | --- |
| Tree planted in depot (Autonomous) | 30 |
| Tree planted in depot (Manual) | 10 |
| All 3 trees planted | +25 completion bonus |

Field Elements

| Element | Specification |
| --- | --- |
| Tree Seedlings | 3 per quadrant, arranged in line at Tree Nursery |
| Planting Depots | 3 separate depot locations, 1 planting hole each |

• Trees must be placed upright in the hole to score.

• Robots may carry only 1 tree at a time.

## M6. Weather Station Deployment

Climate monitoring systems are offline. The robot pushes a weather mast from horizontal to vertical until it latches in place.

Scoring

| Action | Points |
| --- | --- |
| Mast raised and latched upright (Autonomous) | 30 |
| Mast raised and latched upright (Manual) | 10 |

Field Elements

| Element | Specification |
| --- | --- |
| Weather Station | Platform with hinged mast, on track line at side edge |
| Mast | Starts horizontal, latch holds vertical when raised |

• No-equipment constraint: robot cannot be touching the station at match end.

## M7. Seed Bank Collection (Manual Only)

Native plant seeds must be preserved. The robot collects seed containers from track positions and delivers them to the Home Base for safekeeping.

Scoring

| Action | Points |
| --- | --- |
| Each seed container delivered to Home Base(Autonomus) | 25 |
| Each seed container delivered to Home Base( Manual) | 10 |
| All 3 seed containers delivered | +20 completion bonus |

Field Elements

| Element | Specification |
| --- | --- |
| Seed Containers | 3 per quadrant, small cubes on track lines |
| Delivery | Home Base — seeds must be inside Home to score |

## M8. Galamsey Equipment Removal

Illegal mining equipment must be cleared. The robot pushes equipment blocks completely off the field edge.

Scoring

| Action | Points |
| --- | --- |
| Equipment pushed off field (Autonomous) | 30 |
| Equipment pushed off field (Manual) | 10 |

Field Elements

| Element | Specification |
| --- | --- |
| Equipment Blocks | 2 per quadrant, shown as dashed outlines on track positions |
| Push-off Edge | Marked edge of quadrant |

• Blocks must be completely off the field surface to score.

# 4. Shared Center Missions (Manual Only)

The center of the field is shared between all alliances. These missions can only be attempted during the 1-minute manual period.

## M9. AI Command Center

An AI-powered command center sits at the center of the field. Teams must deliver a Data Module, press the Broadcast Button, and raise the Antenna.

Scoring

| Action | Standard | After Accelerator |
| --- | --- | --- |
| Data Module to Upper Slot (120mm) | 30 | 60 |
| Data Module to Lower Slot (40mm) | 10 | 20 |
| Broadcast Button pressed | 15 | 30 |
| First alliance to complete full sequence | +25 | +25 |

Field Elements

| Element | Specification |
| --- | --- |
| Command Center | Central structure at field center, shared |
| Upper Slot | 60 × 60mm at 120mm height |
| Lower Slot | 60 × 60mm at 40mm height |
| Antenna | Hinged arm, starts horizontal |
| Broadcast Button | Spring-loaded, on platform |
| Data Module | 50mm cube, 1 per alliance, starts at Home Base |

• Broadcast Button can only be pressed after a Data Module is inserted.

• Data Module starts at the alliance’s Home Base. Either Robot 1 or Robot 2 may carry and deliver it — the team decides their own strategy.

## M10. Water Purification (Manual Only)

Contaminated water balls are scattered across the field. Robots collect as many as possible and deliver them to the Purification Station, which has an Upper Chamber (harder, more points) and a Lower Chamber (easier, fewer points).

Scoring

| Action | Points |
| --- | --- |
| Water ball to Chamber | 5 |

Field Elements

| Element | Specification |
| --- | --- |
| Water Balls | Multiple, scattered across the field |
| Purification Station | 1 per alliance, at side edge of field |
| Lower Chamber | Opening at 40mm height |

• This mission is MANUAL ONLY.

• Robots may carry any number of water balls they want.

• Each alliance has its own Purification Station on their side.

5. Game Features

## 5.1 Precision Tokens

Each alliance starts with 7 Precision Tokens worth 15 points each (105 total). Each time a team member touches their robot outside the Home Base, one token is removed. Tokens remaining at match end are added to the score. Precision Token stations (PT) are located at various positions around the field edge.

## 5.2 Innovation Point

To earn the Innovation Point, the alliance must autonomously deliver a Data Module to the Upper Slot of the AI Command Center during the 2-minute autonomous period. Either Robot 1 or Robot 2 may attempt this. The Data Module starts at Home Base, so the robot must pick it up and navigate to center autonomously. This is tracked separately in rankings and serves as the first tiebreaker.

| Action | Points |
| --- | --- |
| Data Module to the Upper Slot | 100 |

## 5.3 Emergency Evacuation (Endgame)

When 30 seconds remain, an audible signal sounds. Robots may return to their own Home Base.

| Action | Points |
| --- | --- |
| 1 robot in its Home Base at match end | 10 |
| Both robots in their Home Bases at match end | 35 |

## 5.4 Surprise Rule

A single rule modification is revealed at the Opening Ceremony. Teams get 60 minutes of practice time to adapt before matches begin.

# 6. General Rules

## 6.1 Match Phases

1. Setup (30 sec): Place robots in assigned quadrant Home Base.

2. Autonomous (2 min): Robots in own quadrant only. Line-following, no remote control.

3. Manual (1 min): Full field opens. Remote control. Center missions accessible. Accelerator window: 0:45 to 0:15.

4. Endgame (last 30 sec): Warning signal. Evacuation scoring available.

## 6.2 Autonomous Rules

• Robots must stay in their assigned quadrant during autonomous.

• Crossing the quadrant boundary during autonomous: 10-point penalty.

• Autonomous scoring earns higher points than manual (see each mission).

## 6.3 Penalties

• Touch robot outside Home Base: lose 1 Precision Token

• Cross quadrant boundary during autonomous: 10-point penalty

• Damage field or opponent robot: disqualification

• Wrong color container in pit: -15 points

## 6.4 Tiebreaker Order

1. Higher Autonomous Score

2. Fewer Penalties

3. Innovation Point (autonomous Network Module to center)

4. Replay

# 7. Tournament Format

## 7.1 Preliminary Matches (Ranking Stage)

All preliminary matches are ranking-based. Every team plays 3 preliminary matches. The scores from all 3 matches are added together. Teams are ranked by their total combined score.

Ranking is based purely on the total points earned from completing missions across all preliminary matches. There are no win points — only total score matters.

## 7.2 Qualification to Knockout

The number of teams qualifying to the knockout stage depends on the total number of teams in the league:

| Number of Teams in League | Teams Qualifying to Knockout |
| --- | --- |
| More than 32 teams | Top 32 qualify |
| 17 to 32 teams | Top 16 qualify |
| 9 to 16 teams | Top 8 qualify |
| 8 or fewer teams | All teams may qualify |

Only the highest-ranked teams from the preliminary stage advance. All other teams are eliminated.

## 7.3 Knockout Stage (Direct Elimination)

The knockout stage is direct elimination. Win and advance. Lose and you are out.

In the knockout stage, each match is independent. The only thing that matters is whether you win your specific match — not your score compared to other matches happening elsewhere.

Example: If Team A beats Team B 100–50, and Team C beats Team D 15–10, both Team A and Team C advance. Team B and Team D are eliminated. We do NOT compare Team B’s 50 points to Team C’s 15 points — each match is its own contest.

If a match results in only one team being present (opponent no-show or disqualification), that one team still plays solo and advances to the next stage.

## 7.4 Knockout Tiebreaker

If a knockout match ends in a tie, the following sequence applies:

1. Autonomous Score: Alliance with higher autonomous period score wins

2. Replay: If still tied, a 2-minute replay match is conducted (teams get 5 minutes to prepare)

# 8. Match Procedures

## 8.1 Robot Malfunction

• If a robot is non-functional before the match: the team still reports to the field. The match proceeds with the alliance partner competing alone. Not grounds for replay unless caused by field malfunction.

• If a robot malfunctions during the match: no timeout or repair allowed. Points earned before malfunction still count.

• If both alliance robots are non-functional: opposing alliance plays normally. Non-functional alliance receives 0 points.

## 8.2 Alliance Partner No-Show

• A 2-minute grace period is provided after the match is called

• After grace period: the team is declared a no-show and receives 0 points

• The present team plays alone; their score counts normally

• Repeated no-shows (3 or more) may result in disqualification

## 8.3 Field Malfunction

• Head Referee immediately stops the match

• If less than 1 minute elapsed: match replayed from beginning

• If more than 1 minute elapsed: match continues from point of stoppage

• All teams get 5 minutes to reset robots

## 8.4 Schedule

• Matches scheduled with 5-minute intervals (3-min match + 2-min transition)

• Teams must queue 10 minutes before their scheduled match

• Buffer time built in every 10 matches

• Competition Director has authority to adjust schedules as needed

# 9. Coaches / Adult General Rules

When in doubt, remember: “Student Led, Student Success”

## 9.1 Focus on Learning Over Winning

Emphasize the importance of learning and personal growth over the desire to win. This helps students appreciate the broader skills they develop through the competition.

## 9.2 Respect Student Centeredness

Students must be the only ones designing, building, programming, and troubleshooting their robots. This encourages critical thinking, problem-solving, and teamwork skills.

## 9.3 Provide Guidance, Not Answers

Provide guidance through leading questions and guided discussions rather than providing direct answers or solutions. This helps students develop independence and confidence.

## 9.4 Maintain a Positive Environment

Create a supportive and encouraging atmosphere. Avoid negative comments or criticism. A positive environment boosts student morale and encourages a growth mindset.

## 9.5 Encourage Student Communication

Students should be the only ones who communicate directly with judges, referees, and members of other teams. This gives students opportunities to practice presentation, problem-solving, and negotiation skills.

## 9.6 Stay Behind the Scenes

Adults should stay in the background during active problem-solving and match play. When students are the primary participants and leaders, all teams are on a level playing field.

## 9.7 Respect All Participants

Treat all students, mentors, volunteers, and other event attendees with respect and courtesy. Respectful behavior promotes a fair and enjoyable experience for everyone.

# 10. Complete Scoring Summary

Robot 1 (Top Quadrant)

| Mission | Autonomous | Manual |
| --- | --- | --- |
| M1: Container to correct pit | 40 each | 10 each |
| M1: All 4 pits secured | +30 | +20 |
| M1: Wrong color penalty | -15 | -15 |
| M2: Soil testing lever flipped | 30 | 10 |
| M3: Animal to Home Base | 35 | 10 |
| M4: Accelerator activated | — | 0 (enables 2× M9) |

Robot 2 (Bottom Quadrant)

| Mission | Autonomous | Manual |
| --- | --- | --- |
| M5: Tree planted in depot | 30 each | 10 each |
| M5: All 3 trees planted | +25 | +20 |
| M6: Weather mast raised | 30 | 10 |
| M7: Seed to Home Base | — | 15 each |
| M7: All 3 seeds delivered | — | +20 |
| M8: Equipment off field | 25 each | 15 each |

Shared Center + General

| Action | Points |
| --- | --- |
| M9: Data Module Upper / Lower Slot | 30 / 20 (60 / 40 after Accelerator) |
| M9: Broadcast Button | 15 |
| M9: Antenna raised | 20 |
| M9: First full sequence | +25 |
| M10: Water ball Upper / Lower Chamber | 20 / 10 |
| Endgame: 1 robot home / Both home | 10 / 35 |
| Each Precision Token remaining (7 max) | 15 |
| Maximum token bonus | 105 |

END OF STARS LEAGUE GAME MANUAL v3.0

Ghana Robotics Competition 2026 — Powered by Firefly IO